Radiation Protection Guidelines for β-Ray Thickness Gauges

I. National Radiation Safety Standards

In accordance with China’s Basic Standards for Protection Against Ionizing Radiation and the Safety of Radiation Sources (GB 18871-2002, issued October 8, 2002):

- Occupational Exposure Dose Limits:

  - Annual average effective dose over any consecutive 5 years (no retrospective averaging): 20 mSv.

  - Effective dose in any single year: ≤ 50 mSv.

  - Annual equivalent dose to the lens of the eye: ≤ 150 mSv.

  - Annual equivalent dose to extremities (hands/feet) or skin: ≤ 500 mSv.

II. Comparison of Common Radiation Exposure in Daily Life

- Air travel: 1.1 mSv per 10 hours.

- Medical imaging:

  - Chest X-ray: 1.1 mSv per procedure.

  - Standard radiography: 0.2 mSv per scan.

- CT scans:

  - Brain: 2 mSv.

  - Chest: 8 mSv.

  - Abdomen: 10 mSv.

- CAT angiography: 15 mSv per procedure.

III. Radiation Protection Management Requirements

IV. Radiation Exemption Criteria

A2. Exemptible Sources and Exemption Levels
A radiation source may be exempted from regulatory requirements if approved by authorities under the following conditions:

- Radiation generators or electronic devices (e.g., cathode ray tubes) must meet:

  - Dose rate: ≤ 1 μSv/h (ambient or directional dose equivalent rate at 0.1 m from any accessible surface).

  - Maximum photon energy: ≤ 5 keV.

V. Three Principles of Personal Radiation Protection

- Distance Protection:
Radiation intensity decreases with distance. Maintain ≥1 meter from the source to minimize exposure.

- Shielding Protection:
Use shielding materials (e.g., concrete, iron, or lead) to block radiation.

- Time Protection:
Limit exposure time. Avoid prolonged close contact (>2 hours). Post-exposure recovery includes rest and a high-protein diet.

### VI. Reechi β-Ray Thickness Gauge Protective Enclosures

Reechi β-Ray Thickness Gauge (O-Frame)           Reechi β-Ray Thickness Gauge (C-Frame)